# GIS Numerical Problems - Exam Practice

# Scale and Resolution

## Problem 1.1

A digital map has a scale of 1:25,000 and the pixel size on screen is 2mm × 2mm. Calculate: a) The ground distance represented by one pixel b) The area covered by one pixel in square meters

Solution: a) Ground distance per pixel = 2mm × 25,000 = 50,000mm = 50m b) Area per pixel = 50m × 50m = 2,500 m²

## Problem 1.2

A satellite image has a spatial resolution of 30m × 30m pixels. If you want to create a printed map at 1:50,000 scale where each pixel appears as 1mm × 1mm on paper, calculate: a) How many ground meters does 1mm represent on the printed map? b) How many satellite pixels would fit in 1mm × 1mm on the printed map?

Solution: a) 1mm on map = 50,000mm = 50m on ground b) Satellite pixel = 30m × 30m = 900m² Map pixel = 50m × 50m = 2,500m² Number of satellite pixels = 2,500/900 = 2.78 pixels

# Coordinate Systems and Transformations

## Problem 2.1

Convert the following geographic coordinates to UTM coordinates:

Latitude: 28°36'45"N  Longitude: 77°12'30"E

Zone: 43N (Central Meridian: 75°E)

Given: False Easting = 500,000m, False Northing = 0m, Scale Factor = 0.9996

### Solution Steps:

  - Convert to decimal degrees: Lat = 28.6125°, Lon = 77.2083°

  - Calculate difference from central meridian: ΔLon = 77.2083° - 75° = 2.2083°

  - Apply UTM projection formulas (simplified):

Easting ≈ 500,000 + (ΔLon × 111,320 × cos(Lat) × 0.9996)

Easting ≈ 500,000 + (2.2083 × 111,320 × 0.8746 × 0.9996) ≈ 500,000 + 213,456 = 713,456m

Northing ≈ Lat × 110,540 × 0.9996 ≈ 28.6125 × 110,540 × 0.9996 ≈ 3,168,234m

## Problem 2.2

A point has the reference coordinates: Easting = 645,250m, Northing = 2,890,450m in Zone 44N. Calculate the approximate geographic coordinates if the central meridian is 81°E. Assume same False Easting and False Northing data as stated in the previous problem.

### Solution Steps:

- Remove false easting: True Easting = 645,250 - 500,000 = 145,250m

- Calculate longitude difference: ΔLon = 145,250 ÷ (111,320 × cos(26.1°) × 0.9996) ≈ 1.45°

- Longitude = 81° + 1.45° = 82.45°E

- Latitude = 2,890,450 ÷ (110,540 × 0.9996) ≈ 26.1°N

# Map Projections

## Problem 3.1

Calculate the scale distortion for a Mercator projection at latitude 45°N. Given: True scale at equator = 1:1,000,000

Solution: Scale factor at 45°N = 1/cos(45°) = 1/0.7071 = 1.414 Distorted scale = 1,000,000 × 1.414 = 1:707,107 Scale distortion = 41.4% enlargement

## Problem 3.2

A Lambert Conformal Conic projection has standard parallels at 33°N and 45°N. Calculate the scale factor at 39°N (midpoint).

Solution: For LCC with standard parallels φ₁ = 33° and φ₂ = 45°: At the midpoint φ = 39°, the scale factor is approximately 0.9996 This represents a scale compression of 0.04%

# Satellite-Based Positioning (GPS Errors)

## Problem 4.1

A GPS receiver shows the following error sources:

Satellite clock error: ±2m  Atmospheric delay: ±3m  Multipath error: ±1.5m

Receiver noise: ±0.5m

Calculate:

- The total horizontal positioning error (2D RMS)

- The 95% confidence level error

Solution: a) 2D RMS = √(2² + 3² + 1.5² + 0.5²) = √(4 + 9 + 2.25 + 0.25) = √15.5 = 3.94m b) 95%

confidence = 2 × 3.94m = 7.88m

## Problem 4.2

A differential GPS system has a base station 50km away from the rover.

The base station position error is ±1m, and the differential correction introduces an additional ±0.3m error per 10km distance.

Calculate the expected positioning accuracy at the rover.

Solution: Distance-dependent error = (50km ÷ 10km) × 0.3m = 1.5m Total error = √(1² + 1.5²) = √(1 + 2.25) = √3.25 = 1.8m Expected accuracy = ±1.8m

# Vector Representations and Topology

## Problem 5.1

A polygon has vertices at the following coordinates:

A(100, 200), B(300, 200), C(300, 400), D(100, 400)

Calculate:

- The area of the polygon using the shoelace formula

- The perimeter of the polygon

Solution: a) Area = ½|∑(xᵢyᵢ₊₁ - xᵢ₊₁yᵢ)| = ½|(100×200 - 300×200) + (300×400 - 300×200) + (300×400 - 100×400) + (100×200 - 100×400)| = ½|(-20,000) + (60,000) + (80,000) + (-20,000)| = ½|100,000| = 50,000

square units

b) Perimeter = AB + BC + CD + DA

= 200 + 200 + 200 + 200 = 800 units

## Problem 5.2

Two line segments intersect:

Line 1: from (50, 100) to (150, 200)

Line 2: from (100, 50) to (200, 150)

Find the intersection point coordinates.

Solution: Line 1 equation: y = x + 50 Line 2 equation: y = 0.5x + 0 Setting equal: x + 50 = 0.5x + 0

0.5x = -50

x = -100

This indicates the lines don't intersect within the given segments.

Checking actual segments with parametric equations would show they do intersect at (100, 150).

# Regular Tessellations (Grid Analysis)

## Problem 6.1

A square grid has cell size 25m × 25m. A circular buffer with radius 75m is applied around a point located at grid cell (10, 15).

Calculate:

- How many complete grid cells fall entirely within the buffer?

- The total area covered by the buffer in square meters

Solution: a) Buffer radius = 75m covers 3 × 3 = 9 cells in each direction Complete cells within circle ≈ π × 3² = 28.3 ≈ 28 cells

b) Buffer area = π × 75² = π × 5,625 = 17,671 m²

## Problem 6.2

A triangular irregular network (TIN) has 150 vertices and follows Euler's formula: V - E + F = 2 Where V = vertices, E = edges, F = faces (including outer infinite face)

Calculate the number of edges and triangular faces in the TIN.

Solution: For a TIN: E ≈ 3V - 6 and F ≈ 2V - 5 (for large V) E = 3(150) - 6 = 444 edges F = 2(150) - 5 = 295 faces (including outer face) Triangular faces = 294